Iot based Smart Parking System using Raspberry Pi

Phase 3

Introduction :-

One of the most common problems today is a saturation of parking spaces. Vehicles continue to outnumber existing parking spaces, thus clogging roads. Incidences of violence over occupancy, deformed cars due to a space crunch, and overcharging for parking are some problems that result.

Most cities propose increasing parking spaces to combat the problem. Parks and vacant plots are used as potential parking spaces and multi-level facilities are being built, irrespective of the limited land space and resources. But their exists a silly problem. People enter the parking and then came to know that it's full. Should n't it automated ? Don't you think, we should already know if the parking has space for us or not ? Yeah, isn't it a good thought ? This project will help us show the availability of car slots to park the vehicle. This is implemented by using Raspberry Pi 3 B+ and IR Modules.

Things used in this project

Hardware components :-

- Raspberry Pi 3 Model B

- Modulo IR Transceiver

- Adafruit Standard LCD - 16x2 White on Blue

- Wires

- Soldering kit

Software apps and online services:-

- MQTT

Smart Parking

Raspberry Pi Raspbian

MQTT Setup

# MQTT stands for Message Queuing Telemetry Transport. It is a communication standard for IoT devices. This protocol is an open standard, which means anyone can access it and implement it in a networked application.

# The system of MQTT is not a software package but it can be used by the developers of new software as a standard to send messages to IoT devices and receive responses back. This opens up the possibility of writing new control instruction systems and also creating IoT device monitoring tools.

# The MQTT system was first created in 1999, so it has been around for more than two decades. It was written as a standard to communicate with oil pipeline equipment. As an industrial communication system, MQTT was written within the framework of the Supervisory Control and Data Acquisition (SCADA) framework. The SCADA architecture is designed to communicate with shopfloor equipment in factories and sensors and controllers in process control.

# Step 1: Download MQTT Dashboard App from Playstore

# Step 2: Add broker with the following parameters in your MQTT Dashboard App

# Step 3: Add Toggle to the Project and rename it as slot1 and select ON value as '1' and OFF value as '0'. Select Red color for ON and Green Color for OFF with car icon.

# Step 4: Everything is done, here are the examples of how it works.

# PIN diagram :-

### Schematic for IoT based Smart Parking Sensor

# Raspberry pi

# Python  code :-

import time

import RPi.GPIO as GPIO

import time

import os,sys

from urllib.parse import urlparse

import paho.mqtt.client as paho

GPIO.setmode(GPIO.BOARD)

GPIO.setwarnings(False)

'''

define pin for lcd

'''

# Timing constants

E_PULSE = 0.0005

E_DELAY = 0.0005

delay = 1

# Define GPIO to LCD mapping

LCD_RS = 7

LCD_E  = 11

LCD_D4 = 12

LCD_D5 = 13

LCD_D6 = 15

LCD_D7 = 16

slot1_Sensor = 29

slot2_Sensor = 31

GPIO.setup(LCD_E, GPIO.OUT)  # E

GPIO.setup(LCD_RS, GPIO.OUT) # RS

GPIO.setup(LCD_D4, GPIO.OUT) # DB4

GPIO.setup(LCD_D5, GPIO.OUT) # DB5

GPIO.setup(LCD_D6, GPIO.OUT) # DB6

GPIO.setup(LCD_D7, GPIO.OUT) # DB7

GPIO.setup(slot1_Sensor, GPIO.IN)

GPIO.setup(slot2_Sensor, GPIO.IN)

# Define some device constants

LCD_WIDTH = 16    # Maximum characters per line

LCD_CHR = True

LCD_CMD = False

LCD_LINE_1 = 0x80 # LCD RAM address for the 1st line

LCD_LINE_2 = 0xC0 # LCD RAM address for the 2nd line

LCD_LINE_3 = 0x90# LCD RAM address for the 3nd line

def on_connect(self, mosq, obj, rc):

self.subscribe("Fan", 0)

def on_publish(mosq, obj, mid):

print("mid: " + str(mid))

mqttc = paho.Client()                        # object declaration

# Assign event callbacks

mqttc.on_connect = on_connect

mqttc.on_publish = on_publish

url_str = os.environ.get('CLOUDMQTT_URL', 'tcp://broker.emqx.io:1883')

url = urlparse(url_str)

mqttc.connect(url.hostname, url.port)

'''

Function Name :lcd_init()

Function Description : this function is used to initialized lcd by sending the different commands

'''

def lcd_init():

# Initialise display

lcd_byte(0x33,LCD_CMD) # 110011 Initialise

lcd_byte(0x32,LCD_CMD) # 110010 Initialise

lcd_byte(0x06,LCD_CMD) # 000110 Cursor move direction

lcd_byte(0x0C,LCD_CMD) # 001100 Display On,Cursor Off, Blink Off

lcd_byte(0x28,LCD_CMD) # 101000 Data length, number of lines, font size

lcd_byte(0x01,LCD_CMD) # 000001 Clear display

time.sleep(E_DELAY)

'''

Function Name :lcd_byte(bits ,mode)

Fuction Name :the main purpose of this function to convert the byte data into bit and send to lcd port

'''

def lcd_byte(bits, mode):

# Send byte to data pins

# bits = data

# mode = True  for character

#        False for command

GPIO.output(LCD_RS, mode) # RS

# High bits

GPIO.output(LCD_D4, False)

GPIO.output(LCD_D5, False)

GPIO.output(LCD_D6, False)

GPIO.output(LCD_D7, False)

if bits&0x10==0x10:

GPIO.output(LCD_D4, True)

if bits&0x20==0x20:

GPIO.output(LCD_D5, True)

if bits&0x40==0x40:

GPIO.output(LCD_D6, True)

if bits&0x80==0x80:

GPIO.output(LCD_D7, True)

# Toggle 'Enable' pin

lcd_toggle_enable()

# Low bits

GPIO.output(LCD_D4, False)

GPIO.output(LCD_D5, False)

GPIO.output(LCD_D6, False)

GPIO.output(LCD_D7, False)

if bits&0x01==0x01:

GPIO.output(LCD_D4, True)

if bits&0x02==0x02:

GPIO.output(LCD_D5, True)

if bits&0x04==0x04:

GPIO.output(LCD_D6, True)

if bits&0x08==0x08:

GPIO.output(LCD_D7, True)

# Toggle 'Enable' pin

lcd_toggle_enable()

'''

Function Name : lcd_toggle_enable()

Function Description:basically this is used to toggle Enable pin

'''

def lcd_toggle_enable():

# Toggle enable

time.sleep(E_DELAY)

GPIO.output(LCD_E, True)

time.sleep(E_PULSE)

GPIO.output(LCD_E, False)

time.sleep(E_DELAY)

'''

Function Name :lcd_string(message,line)

Function  Description :print the data on lcd

'''

def lcd_string(message,line):

# Send string to display

message = message.ljust(LCD_WIDTH," ")

lcd_byte(line, LCD_CMD)

for i in range(LCD_WIDTH):

lcd_byte(ord(message[i]),LCD_CHR)

lcd_init()

lcd_string("welcome ",LCD_LINE_1)

time.sleep(0.5)

lcd_string("Car Parking ",LCD_LINE_1)

lcd_string("System ",LCD_LINE_2)

time.sleep(0.5)

lcd_byte(0x01,LCD_CMD) # 000001 Clear display

# Define delay between readings

delay = 5

while 1:

# Print out results

rc = mqttc.loop()

slot1_status = GPIO.input(slot1_Sensor)

time.sleep(0.2)

slot2_status = GPIO.input(slot2_Sensor)

time.sleep(0.2)

if (slot1_status == False):

lcd_string("Slot1 Parked  ",LCD_LINE_1)

mqttc.publish("slot1","1")

time.sleep(0.2)

else:

lcd_string("Slot1 Free  ",LCD_LINE_1)

mqttc.publish("slot1","0")

time.sleep(0.2)

if (slot2_status == False):

lcd_string("Slot2 Parked  ",LCD_LINE_2)

mqttc.publish("slot2","1")

time.sleep(0.2)

else:

lcd_string("Slot2 Free  ",LCD_LINE_2)

mqttc.publish("slot2","0")

time.sleep(0.2)

OUTPUT :-